CAR PARKING SYSTEM

VHDL SIMULATION LAB REPORT

1-) Title And Authors

Lab 1:Car Parking System (VHDL Simulation)

Course: Fundamentals of Electronics / Logic Networks

Authors:

- Francesco Bertaiola

- Stefano Giroletti

- Bekir Hamza Gumus

2-) Objectives and Specifications

The objective of this laboratory activity was to design, implement, and simulate a Car Parking Control System using VHDL and GHDL + GTKWave.
The system detects vehicle arrivals through sensors, processes password inputs, manages multiple cars, and displays appropriate messages on a 7-segment display.

The system must meet the following specifications:

- Asynchronous, active-low reset (reset_n = '0')

- Two digit passwords (password_1, password_2)

- Two entrance sensors: front_sensor, back_sensor

- Three LED indicators:

- Green LED → Successful entry

- Red LED → Wrong password or STOP condition

- 7-segment outputs:

- “En” → Wait for password

- “EE” → Wrong password

- “GO” → Correct password

- “SP” → STOP (Multiple cars)

- STOP mode is triggered when car 2 arrives before car 1 leaves.

The system must return to IDLE after each operation

3-) Block Diagram

Main components:

- FSM (Finite State Machine) implementing IDLE, WAIT_PASSWORD, RIGHT_PASS, WRONG_PASS, and STOP

- Password Comparator

- Counter (10-cycle waiting time)

- 7-segment Encoder

- LED Controller

4-) Components Description

4.1 Finite State Machine (FSM)

Implements 5 states:

- IDLE

- WAIT_PASSWORD

- RIGHT_PASS

- WRONG_PASS

- STOP

- RESET state handling

Transitions depend on:

- Sensor inputs

- Password correctness

- Counter expiration

- passcode timeout

- reset signal

4.2 Counter

Counts 10 clock cycles during password waiting phase.
If the counter reaches 9, the system evaluates the password.

4.3 Password Comparator

Compares password_1 and password_2 with the expected value "01" / "10".

4.4 7-Segment Encoder

Converts symbolic outputs into 7-segment codes:

- E → 0000110

- N → 0101011

- G → 0000010

- O → 1000000

- S → 0100100

- P → 0001100

- All off → 1111111

4.5 LED Driver

- Green LED blinks during RIGHT_PASS

- Red LED blinks during WRONG_PASS and STOP

s

4.6 Timeout Trigger (pswd_in Signal)

This signal is asserted (pswd_in = '1') when the user begins entering the password.
If no correct password is provided within the 10-cycle waiting window, the counter expires and pswd_in automatically returns to '0'.

5-) Simulation and Results

Simulations were performed using GHDL and visualized in GTKWave.
All scenarios required in the laboratory instructions were successfully reproduced.

5.1 Scenario 1 — Normal Entry

Description:
A car arrives (front_sensor='1').
After a 10-cycle waiting time, a correct password ("01", "10") is provided.
The system displays “GO”, green LED blinks, and after back_sensor='1', the system returns to IDLE.

Waveform Evidence: (0 - 1.2 us )

Key observations:

- front_sensor='1' → FSM enters WAIT_PASSWORD

- Counter counts from 0 to 9

- Correct password triggers RIGHT_PASS

- hex_1="0000010", hex_2="1000000" → “GO”

- System returns to IDLE when back_sensor='1'.

5.2 Scenario 2 — Wrong Password

Description:
A car arrives but enters an incorrect password ("00", "00").
The system enters WRONG_PASS, displays “EE”, and the red LED blinks.
A correct password is then entered to exit the error state.

Waveform Evidence: (1.1 – 2.1 us )

Key observations:

- During WAIT_PASSWORD, wrong password applied

- Display shows “EE”

- red_led toggles

- Correct password transitions system to RIGHT_PASS

5.3 Scenario 3 — Multiple Cars (STOP State)

Description:
Car 1 enters with correct password (RIGHT_PASS).
Before Car 1 leaves (back_sensor='1'), Car 2 arrives (front_sensor='1').
The system enters STOP, displaying “SP”, and blinking the red LED.

Waveform Evidence: (2 – 3.2 us )

Key observations:

- Car 1 RIGHT_PASS → GO

- front_sensor='1' and back_sensor='1' simultaneously

- FSM enters STOP

- hex_1="0100100", hex_2="0001100" → “SP”

- After Car 1 leaves (back_sensor='0'), Car 2 can enter normally

5.4 Scenario 4 — Reset

Description:
When reset_n='0' is asserted, the system must immediately return to IDLE regardless of its current state.

Waveform Evidence: (3.15 – 3.4 us )

Observations:

- All outputs reset to default

- Display turns off (1111111)

- LED outputs go low

- FSM re-enters IDLE

6-) Credits

This project was developed by the group using:

- Course material provided by instructor Philippe Velha

- VHDL inspired by lecture materials

Simulation Tools :

- GTKWave screenshots generated by the group

- External diagrams/tools referenced: Antares